## Universal horizontal forging machine with vertical parting dies. Model В1141.

| The machine is intended for hot forming of forgings from cut-to-length blanks or rods in multiple impression dies. The machines can be used for the following technological operations: upsetting, semi-piercing, piercing, reduction, cutting, bending. The horizontal forging machine may operate in the following modes: single strokes, continuous strokes, set up. | Max. diameter of work piece, mm | 140 |
| --- | --- | --- |
|  | Max. diameter of upset flange, mm | 180 |
|  | Upset slide nominal force, kN | 12500 |
|  | Force on clamping slide with dies closed, kN | 12500 |
|  | Upset slide stroke, mm | 460 |
|  | Upset slide stroke with dies closed, mm | 310 |
|  | Slide continuous idle strokes per minute, min | 32 |
|  | Electric motor power, kW | 75 |
|  | Machine overall dimensions, mm |  |
|  | length | 6400 |
|  | width | 4360 |
|  | height Weight | 3760 135000 |