| SHEARBALERS |  | mod | C873P.L1E |
| --- | --- | --- | --- |
| 1- SHEAR |  | Ref. | C.802P |
| - SHEAR CYLINDER |  | Ref. | 2.K3728 |
| - SHEARING FORCE |  | ~ t | 800 |
|  |  | KN | 7740 |
| - SHEAR TROAT WIDTH |  | ~ mm | 950 |
| - SHEAR TROAT HEIGHT |  | ~ mm | 650 |
| - CLAMP |  | Ref. | H18E |
| - CLAMP CYLINDER |  | Ref. | CL1812.03F |
| - CLAMP FORCE |  | ~ t | 100 |
|  |  | KN | 900 |
| - LUBRICATION |  |  | Automatic |
| - SHEAR STROKE (dry, with oil-recirculation) |  | C/1 ‘ | 5-6 |
| 2- PRE-COMPRESSION AND FEEDING BOX |  | Ref. | C2471.33 |
| - “FLAPS” WITH “INTEGRATED” HYGHLY WEAR-RESISTANT STEEL |  | N° | 2 |
| - BOX LENGTH |  | ~ mm | 7100 |
| - OPEN BOX WIDTH |  | ~ mm | 2400 |
| - CLOSED BOX DIMENSIONS | - width | ~ mm | 880 |
| (bale dimensions) | - height | ~ mm | 600 |
| - HYDRAULIC CYLINDERS FOR EACH “FLAP” |  | Ref. | LE1812.F1 |
|  |  | N° | 3 |
| - MAXIMUM SIDE COMPRESSION FORCE (for each “flap”) |  | ~ t | 300 |
|  |  | ~ KN | 3000 |
| 3- FEEDING |  | Ref. | K25F.06 |
| - FEEDING-BALING CYLINDER |  | Ref. | K2518F.D5 |
| - MAXIMUM BALING FORCE |  | ~ t | 160 |
|  |  | KN | 1600 |
| 4- HYDRAULIC UNIT |  | Ref. | L1E.1044 |
| - VARIABLE DISPLACEMENT PISTON PUMPS |  | Ref. | 4 xHPR160D |
| - HIGH SPEED AUXILIARY PUMPS |  | Ref. | - |
| - OIL COOLING AND OIL FILTERING PUMP |  | Ref. | 4D250 |
| - PILOTING PUMP |  | Ref. | 2D30 |
| - TANK CAPACITY |  | Litres | 6000 |
| - “CETOP” STANDARD DIRECTIONAL VALVES |  | Ref. | 10.55 |
| - “CETOP” STANDARD RELIEF VALVES |  | DN | 32 |
| - OVER FLOW VALVES |  | DN | 40 |
| - FULL RETURN-LINE FILTRATION |  | micron | 25 |
| - EXTERNAL AUXILIARY FILTRATION |  | micron | 10 |
| - SEPARATE PILOT CIRCUIT FILTRATION |  | micron | 10 |
| - MAXIMUM PRESSURE |  | BAR | 360 |
| - AIR/OIL COOLING |  | Ref. | 4 x HPA 52 |

| SHEARBALERS |  | mod | C873P.L1E |  |
| --- | --- | --- | --- | --- |
| 5- ELECTRIC POWER UNIT |  | Ref. | IEP 400.4 |  |
| - MAIN ELECTRIC MOTORS |  | kW | 4 x 75 |  |
| - PILOTING MOTOR |  | kW | 4,0 |  |
| - AUXILIARY MOTOR FOR FILTRATION/COOLING |  | kW | 5,5 |  |
| - COOLING FAN MOTOR |  | kW | 4 x 1,1 |  |
| 6- ELECTRONIC COMMAND UNIT |  | Ref. | L1E |  |
| - CONTROL PANEL TO INCLUDE: |  |  | PLC |  |
| - EMERGENCY PUSH-BUTTON |  |  |  |  |
| - MANUAL OR AUTOMATIC MACHINE MODE SELECTOR |  |  |  |  |
| - MANUAL AND AUTOMATIC DRIVE |  |  |  |  |
| - AUTOMATIC CYCLE FOR BALER-LOGGER-SHEAR |  |  |  |  |
| 7- OPERATOR’S CABIN |  | Ref. | 1515.P4 |  |
| 8- LOADING HOPPER ( OPTION ) |  | Ref. |  | K70 |
| - LIFTING CYLINDER |  | Ref. |  | C1210.F1 |
| - HOPPER BOX DIMENSION | - length | mm. |  | 7000 |
|  | - External width | mm. |  | 2300 |
| - INDEPENDENT SUPPORT FRAME |  |  |  |  |
| - LIFTING CONTROL BY PLC |  |  |  |  |

| - Indicative weight of the machine |  | ~ tons | 120 |
| --- | --- | --- | --- |
|  |  | ~ mm | 140 |
| - Shearing capacity (mild steel material) |  | ~ mm | 150 |
|  |  | ~ mm | 75 x 850 |
| - Production rate |  | ~ tons/h | 15-30 |

| SHEARBALER |  |  |  |  | C873P |
| --- | --- | --- | --- | --- | --- |
| - OVERALL DIMENSIONS (indicative) | - length |  | mm |  | 16.500 |
|  | - width |  | mm |  | 350 |
|  | - height |  | mm |  | 4200 |
| Standard fitting |  |  |  |  |  |
| On request |  |  |  |  |  |
| 01 SHEAR |  |  |  |  |  |
| - Cut refusal and cut repetition with steps of about 100 mm |  |  |  |  |  |
| - booster 3” |  |  |  |  |  |
| - Clamp exclusion from the automatic cycle |  |  |  |  |  |
| - Shear over-flow circuit (RSC) |  |  |  |  |  |
| - Shear oil.recirculation circuit (RIG) |  |  |  |  |  |
| - Clamp over-flow circuit (RSP) |  |  |  |  |  |
| - Shear auto-lube |  |  |  |  |  |
| - Regulator of the clamp compression force |  |  |  |  |  |
| - adjustable clamp cylinder cushion |  |  |  |  |  |
| 02 PRE-COMPRESSION AND FEEDING BOX |  |  |  |  |  |
| - Automatic “flaps” opening at the end of the cycle |  |  |  |  |  |
| - Regulator of the “flap” compression force |  |  |  |  |  |
| - Closed “flap” interlock |  |  |  |  |  |
| - adjustable “flap” cylinders cushion |  |  |  |  |  |
| 03 FEEDING UNIT |  |  |  |  |  |
| - adjustable cutting length |  |  |  |  |  |
| - automatic feeding stroke economizer |  |  |  |  |  |
| - Control of the baling force |  |  |  |  |  |
| - Feeding over-flow circuit (RSA) |  |  |  |  |  |
| - adjustable “feed” cylinder cushion |  |  |  |  |  |
| 04 HYDRAULIC UNIT |  |  |  |  |  |
| - valving manifold block |  |  |  |  |  |
| - dual system pressure relief valves |  |  |  |  |  |
| - pressure regulation for each function |  |  |  |  |  |
| - CETOP standards valves |  |  |  |  |  |
| - Permanent magnets |  |  |  |  |  |
| - Visual indicators oil-filter obstruction |  |  |  |  |  |
| - Visual and electric oil level indicator on the tank |  |  |  |  |  |
| - Low oil level interlock |  |  |  |  |  |
| - Pump inlet valve interloock |  |  |  |  |  |
| - Air/oil cooler |  |  |  |  |  |
| - High oil temperature shut-down |  |  |  |  |  |
| - Air filters |  |  |  |  |  |
| - Filtering pilot circuit: 10 micron |  |  |  |  |  |
| - Filtering return-line 25 micron |  |  |  |  |  |
| - Auxiliary Filtering: 10 micron |  |  |  |  |  |
| 05a ELECTRIC POWER UNIT |  |  |  |  |  |
| 05b ELECTRONIC COMMAND UNIT |  |  |  |  |  |
| - PLC controlled |  |  |  |  |  |
| - Electro-hydraulic anti-shock control |  |  |  |  |  |
| - remote control |  |  |  |  |  |
| CYLINDERS |  |  |  |  |  |
| - Hardened and chromed piston rods |  |  |  |  |  |
| - Bronze piston bearings with teflon rings to exclude impurities, for extended life |  |  |  |  |  |

| Certification | CE declaration of conformity. Our machinery meets the European Community Standards (CEE) as specified by the Nations Members of the Community. Instructions and maintenance manual is supplied as specified by CEE standards |
| --- | --- |
| Machine inspection | The machine can be inspected at site in SWITZERLAND ( Can be seen in Operation ). |
| Delivery | Shear will be ready for shipment end of June 2025 ) |
| Transport | ex works Switzerland |
| Exclusions | - Hydraulic oil and lubricants - Foundation - Installation - Connection to the electric current - Transport and transport insurance - Erection, lifting means necessary to unload and commissioning the machine, welding machine - Anything else not specifically listed in this Offer |
| Payment | Down payment: 25% of the total amount with order Balance: before dismantling. |
| Offer validaty | 15 days |